# ESP32 CONTROLLER PER QO100

https://github.com/apozzon/QO-100_RTX_CONTROLLER

|  | QO-100 RTX |  |  |  | IP |  |  |  | SSID |  |  |  | PASSW |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Controller |  |  |  | 192.168.0.214 |  |  |  | SKYAP2 |  |  |  | ************ |  |  |  |
|  | ON/OFF |  | DATV |  | GPSDO |  | … |  | … |  | … |  | … |  | … |  |
|  | V in |  | V pow |  | A pow |  | mW in |  | W out |  | W ref |  | SWR |  | T air-pow |  |
|  | 13.8 |  | 27.5 |  | 1.5 |  | 10 |  | 7,5 |  | 0,6 |  | 1,78 |  | 19-33 |  |

## Misure

| Parametro | Max | PIN | Note |
| --- | --- | --- | --- |
| V ingresso | 13.8 V | 1 | Partitore 1:10 |
| V potenza | 30V | 2 | Partitore 1:10 |
| A potenza | 5A | 3 | ASC712-5A |
| W power in | 5V | 4 |  |
| W power out | 5V | 5 |  |
| W reflected | 5V | 6 |  |
| C temperatura aria | 60C | 11 | DHT22 |
| C P.A. | 60C | 10 | DB18S20 |

## Controlli

| Parametro | Stato | PIN | Note |
| --- | --- | --- | --- |
| On / Off | 0-1 | 7 | Accende e spegne SDR – Arduino sempre on low |
| DATV | 0-1 | 8 | Commuta 12V-18V su Bias-T |
| GPSDO | 0-1 | 9 | Accende GPSDO |

## INSTALLAZIONE IDE E SETTAGGIO BOARD

Setup Steps for Arduino IDE

- Open Arduino IDE and go to File > Preferences.

- Add Board Manager URL: Paste the following into the "Additional Boards Manager URLs" field:
https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json.

- Install Boards: Go to Tools > Board > Boards Manager, search for "esp32" and install the latest version by Espressif Systems.

NB: se non carica support mettere queste due istruzioni in C:\Users\Andrea.DESKTOP-IK5K4JJ\.arduinoIDE\ arduino-cli.yaml

network:

connection_timeout: 300s

- Select Board: Select Tools > Board > ESP32 Arduino > ESP32S3 Dev Module.

- Configure Tools Menu (Crucial for N16R8):

  - USB CDC On Boot: "Enabled" (Allows Serial.print via USB).

  - Flash Size: "16MB (128Mb)".

  - Flash Mode: "QIO 80MHz" or "QOIO 80MHz".

  - Partition Scheme: "16M Flash (3M APP/9.9M FATFS)" or similar large partition.

  - PSRAM: "OPI PSRAM".

- Upload: Connect the board, select the correct Port, and upload a test sketch.

## ARDUINO

Additional board manager URL: https://espressif.github.io/arduino-esp32/package_esp32_index.json

Board scelta : ESP32S3 Dev Module

Librerie

https://github.com/ESP32Async/AsyncTCP

https://github.com/ESP32Async/ESPAsyncWebServer

DHT kxn

JAVA aggiunta libreria Google json

tasto destro su Progetto – Build Path -Add External Archives) – Scegliere file scaricato (es.  gson-2.3.1.jar)

Compilazione

File -> Export

Launch4j

## CONTROL BOARD