# PLUTO, ESP32 and GPSDXO VIA RASPBERRY

Contents

PLUTO, ESP32 and GPSDXO VIA RASPBERRY	1

Preparation of the SD Card for the Pi and basic configuration	2

Mapping a logic channel to Pluto	2

Port Mapping for Raspberry Pi3+ with OS 32 bits	2

Port Mapping for Raspberry Pi3+ with OS 64 bits (untested!!!)	3

Setup the Port forwarding	4

Next step: to connect ESP32 via USB	5

Mapping a logic channel to ESP 32	5

Expose the ESP32 via TCP	6

Next step: connect also GPSDXO via Pi	7

Mapping a logic channel to GPSDXO	7

If you need GPSDXO via TCP	7

GPSDXO service loaded at boot.	8

Note: all IP references are 192.168.0.153, obviously you have to replace them with your specific IP

## Preparation of the SD Card for the Pi and basic configuration

Prepare an SD card for Raspberry Pi with OS 32bits Light using the Raspberry Imager or other system and connect the Pi to the PC via USB

- Connect to the Pi via SSH (for example using Putty COMxx on port 22)

- Login with username and password you defined in the Raspberry Imager

- Install iptables

sudo apt install iptables

sudo apt install iptables-persistent -y

sudo apt install tcpdump -y     (only for tests if something goes wrong)

- Verify the connection of the Pluto using the command “ip a”

- To connect to the Pluto from the PC we have to forward what arriving on the ethernet port (for me 192.168.0.151) of the Pi to the USB port (eth1). To avoid connection conflicts, the port to connect to Pluto cannot be 22 used by the Pi but a different number (for me 23). WEW need to re-route cdata from eth0 (lan) to eth1 (USB. Unfortunately, eth0 and eth1 can swap depending on the start sequence so, before create the port forwarding, it is better to name the device Pluto with a specific name through an UDEV rule (in my case I called it pluto-usb). The rule should work with any Adalm Pluto or Plus Plus because it is recognized by vendor and product ID.

## Mapping a logic channel to Pluto

It is a native ethernet channel between network (Ethernet or Wi-Fi) and the standrad IP port of Pluto (192.168.2.1) visible through a virtual interface (ie  eth0, usb0…). The software (ie SDR Console od Putty on a PC) can access though standard ports (22, 23), 80, 30431 etc.).

## Port Mapping for Raspberry Pi3+ with OS 32 bits

Note: nano editor can be closed with ctrl+X and the file saved following the instructions on screen.

Create a configuration file for UDEV

sudo nano /etc/udev/rules.d/99-pluto-net.rules

insert the following line

SUBSYSTEM=="net", ACTION=="add", ATTRS{idVendor}=="0456", ATTRS{idProduct}=="b673", NAME="pluto-usb"

As alternate write in the specific mac address of your Pluto (you can read it with the command “ip link show dev eth0 or eth1). This solution is much stronger but you cannot change pluto without changing the setup.

SUBSYSTEM=="net", ACTION=="add", ATTR{address}=="00:e0:22:3c:81:90", NAME="pluto-usb"

check that it works

ip a

if not restart the Pi.

## Port Mapping for Raspberry Pi3+ with OS 64 bits (untested!!!)

Create a configuration file

sudo nano /etc/NetworkManager/conf.d/10-pluto-usb.conf

insert the following lines

[device-pluto-usb]

match-device=match-property:ID_VENDOR_ID==0456,match-property:ID_MODEL_ID==b673

set-name=pluto-usb

Disconnect and reconnect UDEV

sudo udevadm control --reload-rules sudo udevadm trigger --attr-match=subsystem=net

Restart the NetworkManager

sudo systemctl restart NetworkManager

check that it works

ip a

if not restart the Pi.

(eth-pluto now pluto-usb to avoid naming convention problems)

## Setup the Port forwarding

Create a configuration file read at the startup

sudo nano /etc/sysctl.d/99-ip-forward.conf

with this line

net.ipv4.ip_forward=1

Clean the NAT table

sudo iptables -t nat -F

# Port Forwarding (Porta 23 SSH, Porta 80 Web, Porta 30431 SDR IIO)

sudo iptables -t nat -A PREROUTING -p tcp --dport 23 -j DNAT --to-destination 192.168.2.1:22

sudo iptables -t nat -A PREROUTING -p tcp --dport 80 -j DNAT --to-destination 192.168.2.1:80

sudo iptables -t nat -A PREROUTING -p tcp --dport 30431 -j DNAT --to-destination 192.168.2.1:30431

# Masquerade verso l'interfaccia fissa del Pluto

sudo iptables -t nat -A POSTROUTING -o pluto-usb -j MASQUERADE

Rewrite the routing table

sudo iptables-save | sudo tee /etc/iptables/rules.v4

That’s it, at restart you can test the connection with Pluto

- via SSH with the same IP address as the Pi but using port 23

- via a web browser using the same IP address on standard port 80

- with SDR Console using the standard port 30431

- any other system…

Other ports to other devices can be forwarded using the same system.

Testing active routing

sudo iptables -t nat -v -L -n --line-numbers

# Connect a GPSDXO via Pi

## Mapping a logic channel to GPSDXO

We need a separate channel to GPSDXO that does not have an IP defined to reach to like Pluto has.

The “socat” command creates a TCP port on the Raspberry and maps it to the serial port of GPSDXO (es. /dev/ttyUSB0).

Since the required connection is only SSH (no TCP) there is a shorter way that does not require the UDEV device definition.

Find the correct port of the GPSDXO (in my case is a Cartrain)

Unplug the GPSDXO and plug it again

ls /dev/ttyUSB* /dev/ttyACM*

Find the hardware details of the connected GPSDXO

lsusb

Giving back something like

Bus 001 Device 004: ID 1a86:7523 QinHeng Electronics CH340 serial converter

idVendor = 1a86

idProduct = 7523

find the proper name

udevadm info -a -n /dev/ttyUSB0 | grep '{serial}'

Add it to the UDEV definition file already in place for Pluto id

sudo nano /etc/udev/rules.d/99-usb-serial.rules

add this line (symlink it’s a symbolic permanet link)

SUBSYSTEM=="tty", ATTRS{idVendor}=="1a86", ATTRS{idProduct}=="7523", SYMLINK+="myGPSDXO"

Or if you have more than one GPSDXO

SUBSYSTEM=="tty", ATTRS{idVendor}=="1a86", ATTRS{idProduct}=="7523", ATTRS{serial}=="B9008xFF", SYMLINK+="myGPSDXO”

Ctrl+X to save

Restart UDEV or restart the Raspberry to make the mod’s active.

sudo udevadm control --reload-rules

sudo udevadm trigger

At this point to connect the GPSDXO from the PC you can just type in CMD

ssh -t pi@<IP_DEL_RASPBERRY> "minicom -D /dev/myGPSDXO -b 115200"

## If you need GPSDXO via TCP

To make GPSDXO reachable from any PC and not only the local host (127.0.0.1), must be modified to listen to all interfaces using the ip 0.0.0.0

Install socat if not there yet (socat: command not found)

sudo apt update

sudo apt install socat -y

then

socat TCP-LISTEN:1080,fork,reuseaddr,bind=0.0.0.0 FILE:/dev/myGPSDXO,b115200,cs8,parenb=0,cstopb=0,raw,echo=0

From this moment all data is sent from GPSDXO to 192.168.0.153:1080 (and vice versa).

You might want to check the flow from GPDDXO to Raspberry with

sudo stty -F /dev/myGPSDXO 115200 raw -echo

sudo cat /dev/myGPSDXO

To see if the data arrives to the PC open Powershell

Test-NetConnection 192.168.0.153 -Port 1080

You’ll get “TcpTestSucceeded : True”

To see the dataflow on the PC use the same Powershell session and launch

$client = [System.Net.Sockets.TcpClient]::new()

$client.Connect("192.168.0.153",1080)

$stream = $client.GetStream()

$reader = [System.IO.StreamReader]::new($stream)

while ($client.Connected) {

$line = $reader.ReadLine()

Write-Host $line

}

Result

If you wish to see them just clicking an icon on the desktop create a .bat file with these instructions (change file name and IP for your use)

@echo off

title TCP Connection to GPSDXO via Raspberry 192.168.0.153

echo Testing Connection... If you do not see "TCP test succeded" something is wrong

powershell  -NoProfile -ExecutionPolicy Bypass -Command "Test-NetConnection 192.168.0.153 -Port 1080"

echo adding data from GPSDXO to file C:\temp\gpsdxo.log

set "LOGFILE=C:\temp\gpsdxo.log"

powershell -NoProfile -ExecutionPolicy Bypass -Command ^

"$client = [System.Net.Sockets.TcpClient]::new(); ^

$client.Connect('192.168.0.153', 1080); ^

$stream = $client.GetStream(); ^

$reader = [System.IO.StreamReader]::new($stream); ^

while ($true) { ^

$line = $reader.ReadLine(); ^

if ($line -eq $null) { break }; ^

$timestamp = Get-Date -Format 'yyyy-MM-dd HH:mm:ss.fff'; ^

$output = $timestamp + '  ' + $line; ^

Write-Host $output; ^

Add-Content -LiteralPath '%LOGFILE%' -Value $output; ^

}; ^

$reader.Close(); ^

$client.Close()"

echo.

echo Connection interrotta o terminata.

pause

```

### GPSDXO service loaded at boot.

To make it valid after shutdown and restart it must be included in a system service created with systemd:

sudo nano /etc/systemd/system/gpsdxo-bridge.service

lines to input

[Unit]

Description=Socat Serial TCP Bridge per GPSDXO

After=network.target

[Service]

Type=simple

User=root

ExecStart=/usr/bin/socat TCP-LISTEN:1080,fork,reuseaddr,bind=0.0.0.0 FILE:/dev/myGPSDXO,b115200,cs8,parenb=0,cstopb=0,raw,echo=0Restart=always

RestartSec=5

[Install]

WantedBy=multi-user.target

Now it has to be read and started and authorized for automatic execution

Reload configuration of  systemd

sudo systemctl daemon-reload

Authorize the automatic execution at startup

sudo systemctl enable gpsdxo-bridge.service

Start the service immediatly

sudo systemctl start gpsdxo-bridge.service

Test restart of the Raspberry and pray.

Check if the service is running

sudo systemctl status gpsdxo-bridge.service

Se serve fai ripartire il servizio con

# 1. Ricarica la configurazione di systemd

sudo systemctl daemon-reload

# 2. Riavvia il servizio bridge

sudo systemctl restart gpsdxo-bridge.service

⚠️ Warning 1⚡

Be sure that the power supplied to Pluto, ESP and GPSDXO will not go all though the Raspberry because it can handle max 1.25A in total (summing up all USB connected consumptions). Pluto cannot be powed though Raspberry!!! It works receiving but get stuck if you try to transmit.

You might want to add a USB Hub to connect your devices.

⚠️ Warning 2

Pluto requires that the GPSDXO signal (40 MHz) is available when turned on instead it gets stuck. Normally GPSDXO generates the RF signal when starts but it may take too long for Pluto and, in any case, the reference frequency is not precise. Pluto uses and memorizes the wrong frequency during the boot sequence, and this disturbs the correct functionality of the Pluto for a long time. To avoid troubles, it is better to delay the Pluto startup after that the Pi and GPSDXO are on.

If you put a timer, remember to cut the red wire of the USB cable from the Pi to the Pluto to avoid Pluto getting power directly from the Pi.

That is not mandatory, but it solves unpredictable problems when you turn the system on.

SOLUTION

Unfortunately, the solution to cut the red wire of the USB data does not work. It is required for the USB handshake, so you have to modify the Pluto cutting wires etc. that is not recommended.

The only way is to have the GPSDXO always on and interrupted only by unplugging the RTX and a timer to start the Pi and the Pluto later.

But, if you modify the Pluto, here is the not recommended solution to be tested!

- Cut the red wire of the USB connection to the Pluto DATA and use a 5V photocoupled relays module connected to the Pi’s GPIO:

VCC Pin 4 or 2

GND Pin 6

IN Signal Pin 11 (GPIO 17)

Connect COM (common) to 5V supply and NO + GND to Pluto POWER USB input.

- Create a Python timer to control the delay the relays when the Pi starts.

nano /home/pi/pluto_start.py

add lines

import time

from gpiozero import OutputDevice

# Wait 1 minute from beginning of the script launched in the startup sequence

time.sleep(60)

# GPIO 17 (Pin 11) configuration (active_high=false in case of inverted logic)

rele = OutputDevice(17, active_high=True, initial_value=False)

# Turn on the relay (hence Pluto)

rele.on()

- Create the system service

sudo nano /etc/systemd/system/pluto-timer.service

add lines

[Unit]

Description=Delayed Pluto boot

After=multi-user.target

[Service]

Type=simple

ExecStart=/usr/bin/python3 /home/pi/pluto_start.py

Restart=on-failure

[Install]

WantedBy=multi-user.target

- Service habilitation to be executed at each boot

sudo systemctl daemon-reload

sudo systemctl enable pluto-timer.service

GL and 73 by I2TPY

# Next step: to connect ESP32 via USB –

# UNTESTED because this functionality could be done by Raspberry itself. Thinking about…

## Mapping a logic channel to ESP 32

We need a separate channel to ESP32 that does not have an IP defined to reach to like Pluto has.

The “socat” command creates a TCP port on the Raspberry and maps it to the serial port of ESP32 (es. /dev/ttyUSB0).

Procedure:

Find the correct port of the ESP32

Unplug the ESP32 and plug it again

ls /dev/ttyUSB* /dev/ttyACM*

Find the hardware details of the connected ESP32

lsusb

Giving back something like

Bus 001 Device 004: ID 0403:6001 Future Technology Devices International, Ltd FT232 Serial (UART) IC

idVendor = 0402

idProduct = 6001

find the proper name

udevadm info -a -n /dev/ttyUSB0 | grep '{serial}'

Add it to the UDEV definition file already in place for Pluto id

sudo nano /etc/udev/rules.d/99-usb-serial.rules

add this line (symlink it’s a symbolic permanett link)

SUBSYSTEM=="tty", ATTRS{idVendor}=="0403", ATTRS{idProduct}=="6001", SYMLINK+="myESP32"

SUBSYSTEM=="tty", ATTRS{idVendor}=="0403", ATTRS{idProduct}=="7523", SYMLINK+="myGPSDXO"

Or if you have more than one ESP32

SUBSYSTEM=="tty", ATTRS{idVendor}=="0403", ATTRS{idProduct}=="6001", ATTRS{serial}=="A9008xFF", SYMLINK+="myESP32”

Ctrl+X to save

Restart UDEV or restart the Raspberry to make the mod’s active.

sudo udevadm control --reload-rules

sudo udevadm trigger

## Expose the ESP32 via TCP

To make ESP32 reachable from any PC and not only the local host (127.0.0.1), socan must be modified to listen to all interfaces using the ip 0.0.0.0

socat TCP-LISTEN:88,fork,reuseaddr,bind=0.0.0.0 FILE:/dev/ttyUSB0,b115200,raw,echo=0

or

socat TCP-LISTEN:88,fork,reuseaddr,bind=0.0.0.0 FILE:/dev/myESP32,b115200,raw,echo=0

From this moment all data sent to 192.168.0.153:88 will go back and forth the ESP32.

To make it valid after shutdown and restart it must be included in a system service created with systemd:

sudo nano /etc/systemd/system/esp32-bridge.service

lines to input

[Unit]

Description=Socat Serial TCP Bridge per ESP32

After=network.target

[Service]

Type=simple

ExecStart=/usr/bin/socat TCP-LISTEN:88,fork,reuseaddr,bind=0.0.0.0 FILE:/dev/myESP32,b115200,raw,echo=0

Restart=always

RestartSec=5

[Install]

WantedBy=multi-user.target

Not it has to be read and started and authorized for automatic execution

Reload configuration of  systemd

sudo systemctl daemon-reload

Authorize the automatic execution at startup

sudo systemctl enable esp32-bridge.service

Start the service immediately

sudo systemctl start esp32-bridge.service

Test restart of the Raspberry and pray.

$client = New-Object System.Net.Sockets.TcpClient("192.168.0.153", 1080); $stream = $client.GetStream(); $reader = New-Object System.IO.StreamReader($stream); while($client.Connected) { $reader.ReadLine() }